TC-ND514A Spec: E/V2.0

Decoding machine

Features

- Support 4-channel HD video decoding output

- Support H.264/265 and other encoding formats decoding

- Supports H.265, H.264 Baseline/Main/High-profile encoding levels

- The whole machine supports 4 HDMI outputs

- Decoding capability: 4-channel 12MP/8-channel 4K/12-channel 5MP/16-channel 4MP/ 20- channel 3 MP/ 32-channel 1080P

- The number of decoding channels of the whole machine is 64

- Supports free division of decoding performance

- HDMI/VGA output resolution: 4k (3840*2160@30Hz) (only port 1 supports in HD mode) , 1920*1080@60Hz, 1280*720@60Hz, 1280*800@60Hz, 1440*900@60Hz, 1280*1024@60Hz, 1366*768@60Hz, 1024*768@60Hz, 800*600@60Hz

- Support video decoding resolution: 3840*2160@30/25fps; 2560*1600@30/25fps; 2048*1536@30/25fps; 1600*1200@30/25fps; 1920*1080@60/50/30/25fps; 1280*720@60/50/30/25fps; 704*576@60/50/30/25fps; 352*288@60/50/30/25fps

- Interactive visual software operation terminal, supports PAD control using Safari browser

- Support adding 64 channels of patrol decoding to a single screen

- Support video file decoding and output on the wall

- Support ONVIF standard protocol to access front-end devices

- Support RTP/RTSP protocol for device preview

- Supports decoding streams from encoding devices using RTSP URL

- Support platform integration of devices via SDK

- Support GB28181 protocol access platform

- Support WEB or platform access, configuration and management

- Support remote acquisition and configuration of parameters, support remote export and import of parameters

- Support remote acquisition of system operation status and system logs

- Support remote restart, restore default configuration, upgrade and other daily maintenance

- The whole machine contains 1 reset button

Technical Specifications:

| Output parameters |  |
| --- | --- |
| Output Interface | 4 HDMI outputs |
| HDMI output resolution | 4k (3840*2160@30Hz) (only supported by port 1 in HD mode) 1920*1080@60Hz, 1280*720@60Hz, 1280*800@60Hz, 1440*900@60Hz, 1280*1024@60Hz, 1366*768@60Hz, 1024*768@60Hz, 800*600@60Hz |
| Basic parameters |  |
| IE Management | Support IE output setting function for each group, alarm input/output function setting, arming and disarming time setting |
| Alarm switch | Support alarm linkage switching |
| Patrol decoding | Support adding 64 channels of patrol decoding to a single screen |
| Video decoding | Support video file decoding and output on the wall |
| Keyboard Control | Support network keyboard RS485 keyboard to control the front end through the decoder |
| Transparent channel | Support transparent channel transmission |
| Network protocol | Supports RTSP , TCP/IP, RTP, UDP, HTTP, IGMP, Telnet, ICMP, ARP and other network protocols |
| Open and compatible | Support ONVIF/GB28181 protocol |
| Management | Support WEB or platform access, configuration and management |
| Configuration | Support remote acquisition and configuration of parameters, support remote export and import of parameters |
| log | Support remote acquisition of system operation status and system logs |
| system maintenance | Support remote restart, restore default configuration, upgrade and other daily maintenance |
| Decoding parameters |  |
| Decoding channels | Single machine 6 4 channels |
| Decoding capability | 4 -channel 1200W@30fps, 8 -channel 800W@30fps, 10 -channel 600W@30fps, 12 -channel 500W@30fps, 16 -channel 400W@30fps, 20 -channel 300W@30fps, 32 -channel 1080P@30fps |
| Video Format | S+265, H.265, H.264, MPEG4 |
| Stream Access Protocol | Private, onvif, rtsp |
| Client streaming | Private, onvif, GB28181 (coordinated with platform streaming) |
| Maximum access bandwidth | 640Mbps​ |
| Interface parameters |  |
| Network Interface | 2 RJ45 10M/100M/1000Mbps adaptive Ethernet ports |
| Audio | not support |
| Call the police | not support |
| Serial Interface | not support |
| USB interface | USB3.0 1, USB2.0 2 (1 on the front panel, 1 on the rear panel) |
| reset button | support |
| Optical port | not support |
| Dual power | not support |
| General Specifications |  |
| power supply | DC12V |
| power | ≤ 20 W |
| Operating temperature | -10℃--＋55℃ |
| Working humidity | 10%--90% |
| size | 441 mm (width) * 341 mm (depth) * 75 mm (height including feet) |
| weight | 4.15 kg |
| Single box size | 540 (L) 175 (W) 455 (H) |
| Single box weight | 5.65 kg |
| Integrated package size | 565(L)485(W)750(H) |
| Integrated package weight | 2 4.6 kg |

Dimensions:

Hardware interface: