TC-ND519A Spec:2/E/V2.0

Decoding machine

Features

- 6/9/12/16-channel decoders now support any combination of mixed cascades, up to 4 units can be cascaded, and can be cascaded with V1.0 version

- Support 4K high-resolution input and output

- Support 2-channel local HD video input + 9-channel video decoding output

- HD video input supports HDMI

- Support H.264/265 and other encoding formats

- Supports H.265, H.264 Baseline/Main/High-profile encoding levels

- Support 16 system plans and quick retrieval

- Audio decoding supports G.711A, G.711U, AAC, ADPCM audio format decoding

- The number of decoding channels of the whole machine is 144

- Supports free division of decoding performance

- Supports 9-screen cascade splicing, window opening, overlay, roaming, cross-screen and other functions

- Supports window opening, overlay, roaming, cross-screen and other functions

- Any signal can achieve stepless zoom function

- Interactive visual software operation terminal, supports PAD control using Safari browser

- Support alarm input/output function setting, support arming and disarming time setting

- Support alarm linkage switching

- Support adding 64 channels of patrol decoding to a single screen

- Support video file decoding and output on the wall

- Support network keyboard/RS485 keyboard to control the front end through the decoder

- Support transparent channel

- HD video input can be segmented, spliced, windowed, roamed, and overlaid along with decoding

- Support ONVIF standard protocol to access front-end devices

- Support RTP/RTSP protocol for device browsing

- Supports decoding streams from encoding devices using RTSP URL

- Support platform integration of devices via SDK

- Support GB28181 protocol access platform

- Support WEB or platform access, configuration and management

- Support remote acquisition and configuration of parameters, support remote export and import of parameters

- Support remote acquisition of system operation status and system logs

- Support remote restart, restore default configuration, upgrade and other daily maintenance

- The whole machine contains 1 reset button

- Display screen compatibility: LCD screen

Technical Specifications:

| Output parameters |  |
| --- | --- |
| Output Interface | 9 HDMI outputs |
| HDMI output resolution | 4k (3840*2160@30Hz) (only supported by ports 1, 5, and 9 in HD mode) 1920*1080@60Hz, 1280*720@60Hz, 1280*800@60Hz, 1440*900@60Hz, 1280*1024@60Hz, 1366*768@60Hz, 1024*768@60Hz, 800*600@60Hz |
| Input parameters |  |
| input interface | 2-way HDMI |
| HDMI input resolution | 4K (3840*2160@30Hz), 1920*1080@60Hz, 1440*900@60Hz, 1366*768@60Hz, 1280*1024@60Hz, 1280*960@60Hz, 1280*800@60Hz, 1280*720@60Hz, 1024*768@60Hz, 800*600@60Hz |
| Splicing capability |  |
| Splicing control function | Supports window opening, overlay, roaming, cross-screen and other functions |
| Splicing capability | 1*2, 1*3, 1*4; 2*1, 2*2, 2*3, 2*4; 3*2, 3*3; 4*2 |
| Window/Roaming | 1*1, 1*2, 1*3, 1*4, 2*1, 2*2, 2*3, 2*4, 3*2, 4*2: 4 windows per screen |
| Basic parameters |  |
| IE Management | Support IE output setting function for each group, alarm input/output function setting, arming and disarming time setting |
| Alarm switch | Support alarm linkage switching |
| Patrol decoding | Support adding 64 channels of patrol decoding to a single screen |
| Video decoding | Support video file decoding and output on the wall |
| Keyboard Control | Support network keyboard RS485 keyboard to control the front end through the decoder |
| Transparent channel | Support transparent channel transmission |
| Network protocol | Support RTSP, TCP/IP, RTP, UDP, HTTP, IGMP, Telnet, ICMP, ARP and other network protocols |
| Open and compatible | Support ONVIF/GB28181 protocol |
| Management | Support WEB or platform access, configuration and management |
| Configuration | Support remote acquisition and configuration of parameters, support remote export and import of parameters |
| log | Support remote acquisition of system operation status and system logs |
| system maintenance | Support remote restart, restore default configuration, upgrade and other daily maintenance |
| Decoding parameters |  |
| Decoding channels | Single machine 144 channels |
| Decoding capability | 12 -channel 1200W@30fps, 24 -channel 800W@30fps, 30 -channel 600W@30fps, 36 -channel 500W@30fps, 48 -channel 400W@30fps, 60- channel 300W@30fps, 96 -channel 1080P@30fps |
| Video Format | S+265, H.265, H.264, MPEG4 |
| audio format | G.711A, G.711U, AAC, ADPCM |
| Audio sampling rate | 8K, 16K, 32K, 48K |
| Stream Access Protocol | Private, onvif, rtsp |
| Client streaming | Private, onvif, GB28181 (coordinated with platform streaming) |
| Number of screen divisions | Non-HD mode: 1*1, 1*2, 1*3, 2*1: 1/2/3/4/6/8/9/10/13/16 2*2, 1*4, 2*3, 2*4, 3*2, 4*2: 1/2/3/4/6/8/9/10/13/16/20/25/36 3*3: 1/2/3/4/6/8/9/10/13/16/20/25/36/40/64/81 HD Mode: 1*1, 1*2, 1*3, 2*1: 1/2/3/4/6/8/9/10/13/16/20/25/36/40/64 |
| Maximum access bandwidth | 640Mbps​ |
| Interface parameters |  |
| Network Interface | 2 RJ45 10M/100M/1000Mbps adaptive Ethernet ports |
| Audio | 1-way 3.5mm audio input interface, 1-way 3.5mm audio output interface |
| Call the police | 8-way alarm input, 8-way alarm output (green terminal) |
| Serial Interface | 2 232 (RJ45*1, green terminal*1); 2 485 (green terminal) |
| USB interface | USB3.0 1, USB2.0 2 (1 on the front panel, 1 on the rear panel) |
| reset button | support |
| Optical port | not support |
| Dual power | not support |
| General Specifications |  |
| power supply | AC 220V |
| power | ≤55W |
| Operating temperature | -10℃--＋55℃ |
| Working humidity | 10%--90% |
| size | 441 mm (width) * 341 mm (depth) * 75 mm (height including feet) |
| weight | 5.25 kg​​ |
| Single box size | 540 (L) 175 (W) 455 (H) |
| Single box weight | 6.55 kg​​ |
| Integrated package size | 565(L)485(W)750(H) |
| Integrated package weight | 28.3kg |

Dimensions:

Hardware interface: